Project Report: Quantitative estimation of energy performance in residential buildings using Regression Models

Submitted by:

Rajesh K Bawa

Executive Summary

Background

There is a growing concern about energy efficiency of the residential buildings. Considerable amount of the research is being done to determine the factors important for energy efficiency, and how they can be used to predict energy efficiency of the buildings

Research question

Based on the previous studies, we select eight important variables, related to building characteristics, which very highly correlated with energy efficiency of the building. We generate simulation dataset of these variables, and use them to predict the energy efficiency of the building. Energy efficiency in this building is measured in terms of heating load

Methods

We use multiple linear regression to model this relationship. Using these variables as default, violates the assumptions of the model. We therefore apply natural log transformation to full model, and then do all possible regression, and stepwise regression and robust regression to choose the best model and best number of variables for an effective predictive model.

Results

Results show that having all variables is not the best way to approach. Getting results from all possible regression and stepwise regression show that 5 variable model, with no relative compactness and no orientation regressor is the best model.

Discussion

This shows us the importance of linear regression in understanding the relationship between dependent and independent variable. This model could be flawed because we see a very high correlation among some independent variables, which leads to collinearity, nevertheless it still gives us the same important predictor as was found by the author using robust regression technique.

This also explains about the importance of glazing area and glazing area distribution in determining the energy efficiency of the building.

Introduction

There has been a growing concern about the energy consumption and its effect on the environment. The most important sources of these energy consumptions are automobiles and households. While we do hear about the fossil fuels and rate at which they are depleting and affecting our environment, rarely do we in open circles talk about household and residential energy consumption. Recent reports from European energy commission2 have suggested that there has been a continuous increase in the household energy consumption. Heating, Ventilation and Air Conditioning (HVAC), which play an integral role in maintaining indoor climate, are the major drivers for this increase in consumption. Due to this growing concern, a considerable amount of research is being done on energy efficiency of buildings and governments are setting mandatory guidelines to regulate household energy consumption. European Union, since 2002 has mandatory energy efficiency guidelines, which every residential building needs to conform to.

For making an energy efficient design, designer needs to have a thorough knowledge of the heating load and cooling load of the building. There is no direct way to measure these variables and these variables heavily depend upon occupancy level, building characteristics, and climate.

In order to study the effect of these variables on energy efficient design, researchers use building simulation software. This helps them to vary one or multiple variables at a time, to see and forecast their effect on the energy efficient design. We in this study use the similar building simulation software created dataset and use regression methods to estimate the best possible relationship between different variables and heating load1. Once, the sufficient confidence is established, engineers and designers can use this relationship to predict heating load in building, just by knowing the information about the variables.

Data

The original paper used Ecotect simulation software to generate 12 building forms. Each building form had same volume but different surface area and dimensions. We used eight variables to permute different combinations of building forms. These variables were: Relative Compactness, Surface Area, Wall Area, Roof Area, Height, Orientation, Glazing area and Glazing area distribution. The Table. 1 shows all the variables used in the study, and the number of possible values each variable can take. The main rationale behind using only these variables and not using other variables like climate and occupancy was that, past studies showed these variables to be highly correlated to heating load and cooling load, and since these variables are just building characteristics and not environmental variables, it is easy for engineer/designer to work around these scenarios.

There were in total 768 observations, and 10 variables. Eight out of these 10 variables were independent variables, and the two were response variables, these being heating load, and cooling load. Even though these values are just generated using simulations, the authors explain them to be close to real life scenarios, and therefore being good indicators of the ground truth. We just use heating load as a response variable in this study.

| S.no | Name of the Variable | No. of possible values | Type of Variable |
| --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 | Relative Compactness Surface Area Wall Area Roof Area Overall Height Orientation Glazing Area Glazing Area Distribution Heating Load (Response 1) | 12 12 7 4 2 4 4 6 586 | Continuous Continuous Continuous Continuous Continuous Continuous Continuous Continuous Continuous |
| 10 | Cooling Load (Response 2) | 636 | Continuous |

Table. 1 List of all the independent and dependent variables in the dataset, and the number of possible values for each variable in the data set. The marked variable (Cooling Load) was not used in the study.

Methods

This section is split into four parts: the first part deals with the exploratory data analysis, the second part deals with analysis of full model, in third part we do all possible regressions and stepwise regression for appropriate variable selection, and in fourth part we do cross-validation using the best selected model and use robust regression (IWLS) to develop an alternative model.

Exploratory data analysis and relationship among variables

This section mostly dealt with plotting the variables and seeing different trends and probability densities of each variable. It is very important to look at both the dependent and independent variables individually and in pairs to check the distribution of the variables and get some idea of relationship between them. We used histograms to estimate the empirical probability densities of each variable. Figure. 1 shows the probability densities of each variable. This step was done to see if all the independent and dependent variables follow the normal distribution.

After getting some idea about the distribution of each variable, we looked at the relationship among variables. This is very important as it gives us some basic idea of the kind of relationship each independent variable has with the response variable, and also among them.  Having a strong correlation among independent variables might lead to problems like collinearity. Collinearity is mostly characterized by increase in standard errors for estimated coefficients. Since most of our variables did not have normal distribution we used Spearman’s rank correlation as a non-parametric test for correlation. The scores of this correlation range between -1 and 1. The signs give us the direction of the relationship, whether the variable is positively or negatively related to another variable, and the magnitude gives us an estimate on strength of relationship. Table. 2 show the estimated correlation coefficient of independent variables with the response variables. Table. 3, is the correlation matrix, showing the correlation coefficients between predictors. We also plotted scatterplot matrix (Figure. 2) to give us some visual context on this computed correlations.

| Variable | Spearman Rank Correlation | p-value |
| --- | --- | --- |
| Relative Compactness Surface Area Wall Area Roof Area Overall Height Orientation Glazing Area Glazing Area Distribution | 0.622134 -0.622134 0.471457 -0.804027 0.8612826 -0.004163 0.3228603 0.0683434 | 0.000000 0.000000 0.000000 0.000000 0.0000000 0.9083001 0.0000000 0.05834211 |

Table. 2 Estimates of the Spearman rank correlation of all independent variables with Heating Load Response Variable

|  |  |  | Relative Compact. | Surface Area | Wall Area | Roof Area | Overall Height | Glazing Area | Orientation | Glazing Area Dist |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rel. Compact |  | 1.0000 |  | -1.0000 | -0.2558 | -0.8708 | 0.8690 | 0.0000 | 0.0000 | 0.0000 |
| Surface Area |  | -1.0000 |  | 1.0000 | 0.2558 | 0.8708 | -0.8690 | 0.0000 | 0.0000 | 0.0000 |
| Wall Area |  |  | -0.2558 | 0.2558 | 1.0000 | -0.1934 | 0.2207 | 0.0000 | 0.0000 | 0.0000 |
| Roof Area | -0.8708 |  |  | 0.8708 | -0.1934 | 1.0000 | -0.9370 | 0.0000 | 0.0000 | 0.0000 |
| Overall Height |  | 0.8690 |  | -0.8690 | 0.2207 | -0.9370 | 1.0000 | 0.0000 | 0.0000 | 0.0000 |
| Glazing Area |  | 0.0000 |  | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 1.0000 | 0.0000 | 0.1875 |
| Orientation |  | 0.0000 |  | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 1.0000 | 0.0000 |
| Glazing Area Dist. |  | 0.0000 |  | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.1875 | 0.0000 | 1.00 |

Table. 3 Correlation matrix showing Spearman correlation between eight input variables. The highlighted values in yellow show some major correlations between different variables.

Regression

We used linear regression to model the relationship between the response variable (heating load) and the predictors. We first ran the non-transformed model, with all variables unchanged or untransformed, and checked the assumptions. The residuals from the model did not follow the normal distribution and did not have constant variance.

Figure. 1 Histograms showing empirical distribution of all variables in the dataset.

We therefore did the log (natural log) transformation of the response variable, which made the data more pliable to the assumptions. We also looked at the interaction between different variables, and added them into the model, but even though some of the interactions were significant, it didn’t particularly improve the adequacy of the model (R2). Moreover, some of the independent variables were highly correlated, which probably made them collinear, and it is normally not advised to add polynomial or interaction terms without correcting for collinearity. We also looked at the problematic data points, which had very high residuals and were not following the assumptions even after transforming response variable. We used different influence measures, like Dfits, and Cook’s Distance to identify these outliers. Following this, we also measured the amount of Multicollinearity in our model, using the variation inflation factors (VIF).

After doing the full model analysis of untransformed and transformed (log transformation of the response) model, we used all possible regression and stepwise regression for appropriate variable selection. All possible regression runs all the possible models 2k, where k is the number of parameters and gives us different statistics to measure the model adequacy. We ran 27 (128) models, and used Mallow’s Cp and adjusted R2, to select three best models. These three models were then fully analyzed, and were checked for assumptions using residual plots. We also looked at different influence measures (Cook’s D and Dfit) to identify major outliers and used VIF (Variance Inflation Factors) to measure Multicollinearity in our model. We also calculated the PRESS statistic, which is considered robust than Mallow’s Cp and adjusted sum of squares to measure model adequacy in these models.

Following all possible regression, we used Stepwise Regression, and employed all three methods: Forward, Backward and Both, and used the Akaike Information Criterion (AIC) to select the best model.

Once the decision on best model and variables to be kept in the model was made, we looked at the cross validation procedure. Cross-validation is an important step, if we are building models to predict some future response. We use K-fold validation method, and used 10 folds to do cross-validation in our models.

Fig. 2 Scatterplot matrix showing visual relationship between both dependent and independent variables.

Results and Discussion

Exploratory Data Analysis

We looked at the empirical distribution of the data (Figure. 1), and could say that none of our variables followed a normal distribution. We used scatterplot matrix (Figure. 2) to see the functional relationship between the variables, and could see some input variables being highly correlated. In order to quantify the magnitude of this association between variables, we used Spearman’s rank correlation test. Table. 1 reports the correlation between predictors and response variables. All variables, except Orientation and Glaze Area Distribution, showed significant correlation with the response variables. We also looked at the relationship between independent variables (Table. 2), and saw that Relative compactness and Surface area are inversely proportional to each other. This, according to authors is an artifact of the way the simulation parameters were used. Other variables, like roof area and surface area and roof area and wall area, also showed strong correlation.

Regression:

We used multiple linear regression using the ordinary least squares method to estimate the relationship between response variable (Heating Load) and its corresponding predictors.

Untransformed Full Model:

Looking at the results from the ANOVA table (Table. 4), all factors (regressors), except orientation were significant. Similar results were obtained when we did the t-test on the individual coefficients of each parameter (Table. 5). The model explained 91.5% of the variation. Looking at the residual plots for the untransformed model (Fig. 3), we found that the model did not follow the assumptions. There were some points, which did not show constant variance, and the data did not follow normal distribution and definitely required transformation.

Full model: HEATING.LOAD ~ REL.COMPACT + S.AREA + W.AREA + O.HEIGHT + GLAZING.AREA + ORIENTATION + GLAZE.AREA.DIS ANOVA Table: Analysis of Variance Table Response: HEATING.LOAD Df Sum Sq Mean Sq F value Pr(>F) REL.COMPACT 1 30238.2 30238.2 3511.8919 < 2.2e-16 *** S.AREA 1 8092.9 8092.9 939.9135 < 2.2e-16 *** W.AREA 1 26144.8 26144.8 3036.4812 < 2.2e-16 *** O.HEIGHT 1 1310.6 1310.6 152.2130 < 2.2e-16 *** GLAZING.AREA 1 5686.0 5686.0 660.3835 < 2.2e-16 *** ORIENTATION 1 0.5 0.5 0.0607 0.8054 GLAZE.AREA.DIS 1 73.1 73.1 8.4940 0.0036 ** Residuals 760 6543.8 8.6 Signif. codes: 0 ‘***’ 0.001 ‘**’ 0.01 ‘*’ 0.05 ‘.’ 0.1 ‘ ’ 1

Table. 4 ANOVA table-showing results for the full-untransformed model.

Table. 5 T-test for coefficients of all parameters in the full-untransformed model.

T-test for Coefficients: Residuals: Min 1Q Median 3Q Max -9.8965 -1.3196 -0.0252 1.3532 7.7052 Coefficients: Estimate Std. Error t value Pr(>¦t¦) (Intercept) 84.014521 19.033607 4.414 1.16e-05 *** REL.COMPACT -64.773991 10.289445 -6.295 5.19e-10 *** S.AREA -0.087290 0.017075 -5.112 4.04e-07 *** W.AREA 0.060813 0.006648 9.148 < 2e-16 *** O.HEIGHT 4.169939 0.337990 12.337 < 2e-16 *** GLAZING.AREA 19.932680 0.813986 24.488 < 2e-16 *** ORIENTATION -0.023328 0.094705 -0.246 0.805 GLAZE.AREA.DIS 0.203772 0.069918 2.914 0.0036** Signif. codes: 0 ‘***’ 0.001 ‘**’ 0.01 ‘*’ 0.05 ‘.’ 0.1 ‘ ’ 1 Residual standard error: 2.934 on 760 degrees of freedom Multiple R-squared: 0.9162, Adjusted R-squared: 0.9154 F-statistic: 1187 on 7 and 760 DF, p-value: < 2.2e-16

Fig. 3 Residual plots for the full-untransformed model. The first plot is for constant variance, and the second one is for test of normal distribution.

Log transformed full model:

Since the untransformed model did not follow the assumptions of constant variance, and normal distributions, we log transformed the response (heating load), and ran the full model. We expected the log transformation to make the data more pliable to the assumptions. Looking at the results, ANOVA table (Table. 6) showed the same results as the untransformed model. All the effects, except orientation were significant. We also did T-test for all the coefficients of the parameters (Table. 7), and results showed that relative compactness, surface area, and orientation were not important, but rest of the factors were important. Looking at the residuals (Fig. 4), log transformation did fix some of the problems, but the data still did not follow the normal distribution. Following this, we calculated some of the influence measures for all our observations, and looked at few problematic observations. Table 8 and Figure 5 shows the problematic observations, and plots of influence measures in the model. Mostly, observation 17 to 20 were the four major problem childs and showed very high residuals and Cook’s D, as compared to rest of the observations.

Full model after log transformation: Log(HEATING.LOAD) ~ REL.COMPACT + S.AREA + W.AREA + O.HEIGHT + GLAZING.AREA + ORIENTATION + GLAZE.AREA.DIS ANOVA Table: Analysis of Variance Table Response: log.heatingLoad Df Sum Sq Mean Sq F value Pr(>F) REL.COMPACT 1 72.331 72.331 4994.20 < 2.2e-16 *** S.AREA 1 11.802 11.802 814.8 < 2.2e-16 *** W.AREA 1 58.255 58.255 4022.3 < 2.2e-16 *** O.HEIGHT 1 5.354 5.354 369.7 < 2.2e-16 *** GLAZING.AREA 1 15.047 15.047 1038.9 < 2.2e-16 *** ORIENTATION 1 0.001 0.001 0.046 0.829 GLAZE.AREA.DIS 1 0.451 0.451 31.1 3.395e-08 *** Residuals 760 11.007 0.014 Signif. codes: 0 ‘***’ 0.001 ‘**’ 0.01 ‘*’ 0.05 ‘.’ 0.1 ‘ ’ 1

Table. 6 ANOVA table for log transformed full model.

T-test for Coefficients: Residuals: Min 1Q Median 3Q Max -0.35472 -0.05718 0.00176 0.06585 0.31204 Coefficients: Estimate Std. Error t value Pr(>¦t¦) (Intercept) -0.5787874 0.7806251 -0.741 0.4587 REL.COMPACT 0.5623864 0.4220009 1.333 0.1830 S.AREA 0.0014251 0.0007003 2.035 0.0422 * W.AREA 0.0015991 0.0002727 5.865 6.70e-09 *** O.HEIGHT 0.2665273 0.0138620 19.227 < 2e-16 *** GLAZING.AREA 1.0117131 0.0333840 30.305 < 2e-16 *** ORIENTATION -0.0008389 0.0038841 -0.216 0.8290 GLAZE.AREA.DIS 0.0159934 0.0028675 5.577 3.39e-08 *** Signif. codes: 0 ‘***’ 0.001 ‘**’ 0.01 ‘*’ 0.05 ‘.’ 0.1 ‘ ’ 1 Residual standard error: 0.1203 on 760 degrees of freedom Multiple R-squared: 0.9368, Adjusted R-squared: 0.9362 F-statistic: 1610 on 7 and 760 DF, p-value: < 2.2e-16

Table. 7 T-test for coefficients of all parameters for log transformed model.

Fig. 4 Residual plots for full log transformed model.

S.No. 	        Hat matrix (H)	         Deleted Residuals	             Cook’s D		Dfits

17	        0.0286945			3.675855		              0.02804		0.6318

18	        0.0286945			4.131596		  	0.03526		0.7101

19	        0.0286945			4.098820		  	0.03472		0.7044

20	        0.0286945			3.766553		 	0.02942		0.6473

Table. 8 Influence measures of some of the unusual observations from log transformed model.

Fig. 5 Influence measure plots for all observations from full log transformed model.

Following all the above analysis, we measure Multicollinearity in our model. If the regressors are correlated, it can inflate the variation, and increase collinearity in the model, making some of the significant factors, not significant. Therefore, it is very important to check the Multicollinearity in the regression model. We found severe issues of Multicollinearity in our regressors, especially relative compactness and surface area. Overall height factor also showed very high value for VIF (variation inflation factors). Table 9 shows the Multicollinearity for all the regressors in terms of VIF’s.

FACTORS                						VIF

REL.COMPACT        						105.524054

S.AREA             							201.531134

W.AREA               						   7.492984

O.HEIGHT            						 31.205474

GLAZING.AREA       	 					   1.047508

ORIENTATION          		             				   1.000000

GLAZE DISTRIB.  	  					   1.047508

Table. 9 Multicollinearity issues in regressors of full log transformed model. The highlighted values show alarmingly high values of VIF’s.

Variable selection Techniques:

After analyzing the full model and transformed full model, we looked at two different variable selection techniques. The first one we looked at was all possible regression. We ran 128 (27) models and used Mallow’s Cp and adjusted R2 to select three best models, other than the full model. The three models showed very low value of Cp and very high-adjusted R2. The first model had 5 variables and the next two models had 6 variables. All of these models were then fully analyzed, and we looked at problematic observations, influence measures and problems related to Multicollinearity. Table 10 shows the top 20 results from all possible regressions.

| S.No | No. of parameters | Rel. Compact | S. Area | W. Area | O. Height | Orient | Glazing Area | Glaze. Area.Dist | CP | Adj. R2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 |  |  |  | * |  |  |  | 1572.94 | 0.80 |
| 2 | 1 |  | * |  |  |  |  |  | 5832.06 | 0.45 |
| 3 | 1 | * |  |  |  |  |  |  | 6272.94 | 0.41 |
| 4 | 2 |  |  |  | * |  | * |  | 536.01 | 0.89 |
| 5 | 2 |  |  | * | * |  |  |  | 1076.49 | 0.85 |
| 6 | 2 |  | * |  | * |  |  |  | 1134.91 | 0.84 |
| 7 | 3 |  |  | * | * |  | * |  | 39.55 | 0.93 |
| 8 | 3 |  | * |  | * |  | * |  | 97.98 | 0.93 |
| 9 | 3 | * |  |  | * |  | * |  | 165.48 | 0.92 |
| 10 | 4 | * |  | * | * |  | * |  | 10.44 | 0.93 |
| 11 | 4 |  | * | * | * |  | * |  | 34.93 | 0.93 |
| 12 | 4 |  |  | * | * |  | * | * | 37.29 | 0.93 |
| 13 | 5 | * |  | * | * |  | * | * | 5.82 | 0.94 |
| 14 | 5 |  |  | * | * | * | * | * | 8.18 | 0.94 |
| 15 | 5 |  | * | * | * |  | * | * | 12.4 | 0.94 |
| 16 | 6 | * | * | * | * |  | * | * | 6.04 | 0.94 |
| 17 | 6 |  | * | * | * | * | * | * | 7.77 | 0.94 |
| 18 | 6 | * |  | * | * | * | * | * | 10.14 | 0.94 |
| 19 | 7 | * | * | * | * | * | * | * | 8.00 | 0.94 |

Table. 10 Top 19 results from all possible regression runs. The filled cells with ‘*’ are the variables in the models. Highlighted cells were the models selected for further analyses.

Analysis of the best regression models:

First model:

(Log (HEATING.LOAD) ~ REL.COMPACT + W.AREA + O.HEIGHT + GLAZING.AREA + GLAZE.AREA.DIS)

In this model we removed surface area and orientation regressors from the model. This model had the lowest Mallow’s Cp and very high-adjusted R2, same as the full model. So, we don’t loose any information in terms of amount of variation explained by the model. Looking at the results, everything in the model was significant. In the t-test for coefficients, relative compactness was not as significant as rest of the factors. The residuals still showed major deviation from the normal distribution, with some of the points being the major outliers. We looked at the influence measures, and selected observations showing very high Cook’s D and studentized residuals. The Multicollinearity was not a major issue in this model. Relative compactness and overall height were two factors with VIF close to 10, but there was major reduction in VIF from 100 and 200 values to what we saw here. The model explained 94% of the variation in the data, and had a lowest Mallow’s Cp of 5.82. Tables below show the full analysis of this model.

Model: Log(HEATING.LOAD) ~ REL.COMPACT + W.AREA + O.HEIGHT + GLAZING.AREA + GLAZE.AREA.DIS ANOVA Table: Analysis of Variance Table Response: log.heatingLoad Df Sum Sq Mean Sq F value Pr(>F) REL.COMPACT 1 72.331 72.331 4979.904 < 2.2e-16 *** W.AREA 1 60.915 60.915 4193.908 < 2.2e-16 *** O.HEIGHT 1 14.436 14.436 993.911 < 2.2e-16 *** GLAZING.AREA 1 15.047 15.047 1035.962 < 2.2e-16 *** GLAZE.AREA.DIS 1 0.451 0.451 31.018 3.545e-08 *** Residuals 762 11.068 0.015 Signif. codes: 0 ‘***’ 0.001 ‘**’ 0.01 ‘*’ 0.05 ‘.’ 0.1 ‘ ’ 1

Table. 11 ANOVA table for the first best model chosen using all possible regression.

Residuals: Min 1Q Median 3Q Max -0.35526 -0.05936 0.00016 0.05896 0.31490 Coefficients: Estimate Std. Error t value Pr(>¦t¦) (Intercept) 0.9911257 0.1086741 9.120 < 2e-16 *** REL.COMPACT -0.2578535 0.1251231 -2.061 0.0397 * W.AREA 0.0020209 0.0001774 11.394 < 2e-16 *** O.HEIGHT 0.2430700 0.0077101 31.526 < 2e-16 *** GLAZING.AREA 1.0117131 0.0334319 30.262 < 2e-16 *** GLAZE.AREA.DIS 0.0159934 0.0028717 5.569 3.54e-08 *** Signif. codes: 0 ‘***’ 0.001 ‘**’ 0.01 ‘*’ 0.05 ‘.’ 0.1 ‘ ’ 1 Residual standard error: 0.1205 on 762 degrees of freedom Multiple R-squared: 0.9365, Adjusted R-squared: 0.9361 F-statistic: 2247 on 5 and 762 DF, p-value: < 2.2e-16

Table. 12 T-test for coefficients of all the parameters of the first best model chosen using all possible regression.

Fig. 6 Residual plots for the first model from all possible regressions.

S.No. 	        Hat matrix (H)	         Deleted Residuals	             Cook’s D		Dfits

2	        0.0184651			2.436778	              		0.018497		-0.3342

3	        0.0184651			2.436778		  	0.018497		-0.3342

4	        0.0184651			2.436778		  	0.018497		-0.3342

Table. 13 Influence measures of the three major problematic observations in first model of all possible regressions. Even though there were many problematic observations, these had the highest value of studentized residuals.

Fig. 7 Influence measure plots for all observations in first model from all possible regressions.

FACTORS                						    	 VIF

REL.COMPACT        						   	 9.250283

W.AREA               						  	 3.161934

O.HEIGHT            						  	 9.626103

GLAZING.AREA       	 					  	 1.047508

GLAZE DISTRIB.  	  					  	 1.047508

PRESS Statistic: 11.25835

Adjusted R2: 0.9361

Table. 14 Measures of Multicollinearity in the first model chosen from all possible regressions. We also stated different measures of model adequacy in the table.

Second Model:

(Log (HEATING.LOAD) ~ REL.COMPACT + S.AREA + W.AREA + O.HEIGHT + GLAZING.AREA + GLAZE.AREA.DIS)

This was the second best model in terms of Mallow’s Cp and adjusted R2 reported in all possible regression. The model contains 6 regressors, and we removed orientation factor from it. Looking at the results, in ANOVA table, all the factors/regressors were significant, but in the t-test for coefficients, relative compactness was not significant, and surface area was barely significant. The residuals again did not look good. There were some major deviations from normality and some the points were beyond the limits of -2 +2 of deleted residuals in constant variance plots. This model had some major Multicollinearity issues as well. Relative compactness and surface area are in perfect correlation to each other, which inflates the variation, and therefore a very high VIF’s were reported for these factors. We also looked at some of the major problem children in the data, and calculated Dfit and Cook’s D for all our observations. The model explained 94% of variation to the data. Tables and figures below show the complete analysis of the model.

Model: Log(HEATING.LOAD) ~ REL.COMPACT + S.AREA + W.AREA + O.HEIGHT + GLAZING.AREA + GLAZE.AREA.DIS ANOVA Table: Analysis of Variance Table Response: log.heatingLoad Df Sum Sq Mean Sq F value Pr(>F) REL.COMPACT 1 72.331 72.331 5000.465 < 2.2e-16 *** S.AREA 1 11.802 11.802 815.904 < 2.2e-16 *** W.AREA 1 58.255 58.255 4027.330 < 2.2e-16 *** O.HEIGHT 1 5.354 5.354 370.150 < 2.2e-16 *** GLAZING.AREA 1 15.047 15.047 1040.240 < 2.2e-16 *** GLAZE.AREA.DIS 1 0.451 0.451 31.146 3.328e-08 *** Residuals 761 11.008 0.014 Signif. codes: 0 ‘***’ 0.001 ‘**’ 0.01 ‘*’ 0.05 ‘.’ 0.1 ‘ ’ 1

Table. 15 ANOVA table for the second model that was chosen using all possible regressions.

Residuals: Min 1Q Median 3Q Max -0.35597 -0.05759 0.00207 0.06513 0.31246 Coefficients: Estimate Std. Error t value Pr(>¦t¦) (Intercept) -0.5817237 0.7800176 -0.746 0.4560 REL.COMPACT 0.5623864 0.4217365 1.334 0.1828 S.AREA 0.0014251 0.0006999 2.036 0.0421* W.AREA 0.0015991 0.0002725 5.869 6.55e-09 *** O.HEIGHT 0.2665273 0.0138533 19.239 < 2e-16 *** GLAZING.AREA 1.0117131 0.0333631 30.324 < 2e-16 *** GLAZE.AREA.DIS 0.0159934 0.0028657 5.581 3.33e-08 *** Signif. codes: 0 ‘***’ 0.001 ‘**’ 0.01 ‘*’ 0.05 ‘.’ 0.1 ‘ ’ 1 Residual standard error: 0.1203 on 761 degrees of freedom Multiple R-squared: 0.9368, Adjusted R-squared: 0.9363 F-statistic: 1881 on 6 and 761 DF, p-value: < 2.2e-16

Table. 16 t-test for coefficients for all parameters of the second model that was chosen using all possible regressions.

FACTORS                						    	 VIF

REL.COMPACT        						   	 105.5240

S.AREA									 201.5311

W.AREA               						  	 7.492984

O.HEIGHT            						  	 31.20547

GLAZING.AREA       	 					  	 1.047508

GLAZE DISTRIB.  	  					  	 1.047508

PRESS Statistic: 11.21515

Adjusted R2: 0.9363

Table. 17 Measures of Multicollinearity in the second model chosen from all possible regressions. We also stated different measures of model adequacy in the table.

Fig. 8 Residual plots for the second model from all possible regressions.

S.No. 	     Deleted Residuals	         Hat Matrix (H)	             Cook’s D		Dfits

2	        2.4794			0.01878              		0.016701		-0.34307

3	        2.4794			0.01878		  	0.016701		-0.34307

4	        2.4794			0.01878	  		0.016701		-0.34307

16	        2.9931			0.01193			0.015299		-0.32895

38	        2.9718			0.00961			0.012126		-0.29284

39	        2.9718			0.00961			0.012126		-0.29284

Table. 18 Influence measures of the three major problematic observations in the second model of all possible regressions. Even though there were many problematic observations, these had the highest value of studentized residuals.

Fig. 9 Influence measure plots for all observations in first model from all possible regressions.

Third Model:

(Log (HEATING.LOAD) ~ S.AREA + W.AREA + O.HEIGHT + ORIENTATION + GLAZING.AREA + GLAZE.AREA.DIS)

This was the third best model chosen from all possible regressions. In this we removed relative compactness regressor from the model. It has a Mallow’s Cp of 7.77 and explained nearly 94% of variation in the data. Looking at the results, both ANOVA table and t-test for coefficients gave the same results. The orientation was not significant, and surface area was barely significant, but everything else was very significant. Looking at the collinearity issues, this had some major collinearity issues as well. Maybe not as high as the last model, but higher values around 18 and 20. Surface area and Overall Height were the two regressors, which showed highest VIF in the model. Residual plots also didn’t look very good. Even though most of the points showed constant variance, the normality of the residuals was the major issue here. We also looked at major influential data points among all the outliers, especially the ones with highest values of studentized residuals. Tables below show the results for this model.

Analysis of Variance Table Response: log.heatingLoad Df Sum Sq Mean Sq F value Pr(>F) S.AREA 1 78.716 78.716 5429.5406 < 2.2e-16 *** W.AREA 1 60.728 60.728 4188.7740 < 2.2e-16 *** O.HEIGHT 1 8.272 8.272 570.5732 < 2.2e-16 *** GLAZING.AREA 1 15.047 15.04 1037.8781 < 2.2e-16 *** ORIENTATION 1 0.001 0.001 0.0466 0.8291 GLAZE.AREA.DIS 1 0.451 0.451 31.0758 3.447e-08 *** Residuals 761 11.033 0.014 Signif. codes: 0 ‘***’ 0.001 ‘**’ 0.01 ‘*’ 0.05 ‘.’ 0.1 ‘ ’ 1

Table. 19 ANOVA table for third model chosen using all possible regressions.

Residuals: Min 1Q Median 3Q Max -0.35543 -0.06025 0.00184 0.06121 0.31122 Coefficients: Estimate Std. Error t value Pr(>¦t¦) (Intercept) 0.4457693 0.1354165 3.292 0.00104 ** S.AREA 0.0005337 0.0002075 2.572 0.01029 * W.AREA 0.0018063 0.0002241 8.061 2.92e-15 *** O.HEIGHT 0.2547154 0.0106635 23.887 < 2e-16 *** GLAZING.AREA 1.0117131 0.0334010 30.290 < 2e-16 *** ORIENTATION -0.0008389 0.0038861 -0.216 0.82914 GLAZE.AREA.DIS 0.0159934 0.0028690 5.575 3.45e-08 *** Signif. codes: 0 ‘***’ 0.001 ‘**’ 0.01 ‘*’ 0.05 ‘.’ 0.1 ‘ ’ 1 Residual standard error: 0.1204 on 761 degrees of freedom Multiple R-squared: 0.9367, Adjusted R-squared: 0.9362 F-statistic: 1876 on 6 and 761 DF, p-value: < 2.2e-16

Table 20. T-test for coefficients for third best model chosen using all possible regression.

Fig.10 Residual plots for model three, chosen using all possible regression.

FACTORS                						    	 VIF

S.AREA									 17.6666

W.AREA               						  	  5.0556

O.HEIGHT            						  	 18.4474

ORIENTATION								   1.0000

GLAZING.AREA       	 					  	   1.0475

GLAZE DISTRIB.  	  					  	   1.0475

PRESS Statistic: 11.25208

Adjusted R2: 0.9362

Table. 21 Measures of Multicollinearity in the model three chosen using all possible regression. Highlighted values show very high VIF’s. We also stated PRESS statistic and adjusted R2 for the model.

S.No. 	     Deleted Residuals	         Hat Matrix (H)	             Cook’s D		Dfits

16	        2.988613			0.01425886		0.01826669	-0.3594435

38	        2.988613			0.00983934		0.01248312	-0.2971316

39	        2.97352			0.00983934		0.01242424	-0.2964218

Table. 22 Influence measures of the three major problematic observations in the third model of all possible regressions. Even though there were many problematic observations, these had the highest value of studentized residuals.

Fig. 11 Influence measure plots for model three, chosen using all possible regression.

Stepwise Regression: We also used stepwise regression to select the appropriate variables for the model. Mostly, one chooses one or the other tests for the variable selection, but we wanted to see the effectiveness of both methods, and if they would converge to same conclusion. We used all three methods: backward, forward, and both to run stepwise regression.

Backward:

In backward stepwise regression we start with the full model, and remove each variable at a time to estimate its effect on model adequacy. In our method, stepwise regression used AIC, akaike information criterion, to do that. The results showed that five variable model, with relative compactness and orientation removed will be the best model. This is exactly the same first model we got from the all-possible regression. Table below shows the ANOVA results for backward regression. For detailed results, refer to supplementary information.

ANOVA Table:

Step 			Df     	Deviance 	Resid.Df 	Resid. Dev       AIC

1               		NA           NA       	760   		11.00704 	-3244.356

2 - ORIENTATION  	  1 	0.000675       	761   		11.00772 	-3246.308

3 - REL.COMPACT  	  1 	0.025721       	762   		11.03344 	-3246.516

Table. 23 ANOVA table for the backward regression performed on the full model.

Forward:

In forward stepwise regression we start with the intercept, and add on model based on increase in model adequacy. Here again AIC was the criteria used to measure model adequacy. The results were same as backward regression, a five variable model with no relative compactness or orientation. Table below shows the ANOVA results for forward regression.

Step  		Df     	Deviance 	Resid. Df 	Resid. Dev       	AIC

1          			NA           NA        	767  		174.24644 	-1137.189

2   + O.HEIGHT 	-1 	140.4006    	766   		33.84580 	-2393.677

3   + GLAZING.AREA 	-1  	15.04686    	765   		18.79894 	-2843.272

4   + W.AREA 		-1   	  7.21903      	764   		11.57990 	-3213.390

5   + GLAZE.AREA.DIS 	-1   	   0.45052     	763   		11.12937 	-3241.867

6         + S.AREA 	-1   	   0.09593     	762   		11.03344 	-3246.516

Table. 24 ANOVA table-showing results of forward stepwise regression.

Both:

In this type of stepwise regression, we use combination of both method forward and backward to get to the best model. This starts with the full model and continues to add and remove regressors based on increase or decrease in model adequacy. We obtained same results in this method, as we did for other two ones explained above. It was the same first model from all possible regression, with lowest Mallow’s Cp, best for this study. Table below shows the ANOVA table for ‘both’ stepwise regression.

Step 		Df     Deviance 		Resid. Df 	Resid. Dev       	AIC

1               		NA           NA      		 760   		11.00704 	-3244.356

2 - ORIENTATION  	1 	0.000675   	 761   		11.00772 	-3246.308

3 - REL.COMPACT  	1 	0.025721  	 762   		11.03344 	-3246.516

Table. 25 ANOVA table showing results for ‘both’ stepwise regression.

For full results, please refer to supplementary information at the back of the paper.

Cross-Validation:

Following all the above analysis, we had our best model, and our best variables, so therefore we wanted to look at the prediction ability of it. We used 10-fold cross validation procedure to perform cross-validation in our data set. The benefit of doing the cross-validation is that it is best way to address over fitting of the models. It can be used in conjuction with looking at the residuals and give us some idea of predictive ability of the model. We used DAAG package in R, and used CVlm function to do 10-k cross validation in our data-set. The k – fold method randomly removes k – folds for the testing set and models the remaining (training set) data. Looking at the results, it seems like the model had a decent predictive power. There are lots of instances where the prediction deviates from the observed reading, and this is the case of using same data set as training and test data. Once, this goes to predicting outside the data, there might be some rapid decay in the predicting ability, and this is probably attributed to the model not following normality very well. Figure below shows the results of 10-fold cross-validation in my data set.

Fig. 12 Results from the 10-fold cross-validation using the best chosen model in our data set. Different colors show differen k’s of validation.

Conclusion

In this study we explored the relationship between eight independent variables related to building characteristics and energy efficiency (heating load). We show here that all the regressors are not the good predictors for energy efficiency. In the case of studies, which use simulation software to generate data, there can be an issue of correlation among the variables, and we saw that in this study. Relative compact and Surface Area were in perfect negative correlation; therefore having both of them was leading to increase in variance in the models, and problems relating to collinearity. Orientation in general was not an important regressor. It came out insignificant in most of the models presented here. Therefore, removing these two regressors made sense.

The other important problem, which needed to be addressed, was the problem of residuals. The residuals even after choosing the best model do not look good. The normality is the key assumption in the OLS based regression, and our data-set even after transforming regressors and response was not getting pliable to our assumption. In this case the best case scenario is to use generalized linear models. We did try robust regression in our data set, which uses iteratively reweighted least squares method to do the prediction. Based on our results, and the collinearity in the robust regression model (please refer to supplementary info for details.), the OLS model came out better. OLS model had lower VIF’s than robust regression model. But since assumptions are a key criteria, things like these should be interpreted with caution.

In the end, these analyses explain about the power of the linear model, and not to take them as lightly. As, explained in the class, even though linear models approximate all relationships as linear, and therefore could be the over-simplification of the problem, it at the same time could be very powerful with respect to the application of it in the wider scenario. As the George Box quote says “All models are wrong, some are useful”, this flawed model explained us about the importance of different building characteristics based variables in predicting the heating load of the building.

Best predicted model was:

Heating Load = 0.99 -0.257*REL.COMPACT + 0.002*Wall Area + 0.243*O.Height

+ 1.011713*Glaze Area + 0.0159*Glaze Area Dist

References

- A. Tsanas, A. Xifara: 'Accurate quantitative estimation of energy performance of residential buildings using statistical machine learning tools', Energy and Buildings, Vol. 49, pp. 560-567, 2012

- European Commission, Directive 2002/91/EC of the European Parliament and of the Council of 16th December 2002 on the energy performance of buildings, Official journal of the European Communities, L1/65–L1/71, 04/01/2003.

Appendix/Supplementary Information

Data Source: Data can be downloaded from http://archive.ics.uci.edu/ml/datasets/Energy+efficiency

Regression Outputs and Plots:

STEPWISE REGRESSION:

Backward:

Start:  AIC=-3244.36

log.heatingLoad ~ REL.COMPACT + S.AREA + W.AREA + O.HEIGHT +

GLAZING.AREA + ORIENTATION + GLAZE.AREA.DIS

Df 	Sum of Sq    	RSS     		AIC

- ORIENTATION      1    	0.0007 		11.008 		-3246.3

- REL.COMPACT       1    	0.0257 		11.033 		-3244.6

<none>                       			11.007 		-3244.4

- S.AREA          	 1   	 0.0600 	11.067 		-3242.2

- GLAZE.AREA.DIS   1    	 0.4505 	11.458 		-3215.5

- W.AREA          	 1    	 0.4982 	11.505 		-3212.4

- O.HEIGHT        	 1    	 5.3541 	16.361 		-2941.9

- GLAZING.AREA     1   	 13.3013 	24.308 		-2637.9

Step:  AIC=-3246.31

log.heatingLoad ~ REL.COMPACT + S.AREA + W.AREA + O.HEIGHT +

GLAZING.AREA + GLAZE.AREA.DIS

Df 	Sum of Sq    	RSS     		AIC

- REL.COMPACT     	1    	0.0257 		11.033 		-3246.5

<none>                        			11.008 		-3246.3

- S.AREA          	1    	0.0600 		11.068 		-3244.1

- GLAZE.AREA.DIS  1          0.4505 		11.458 		-3217.5

- W.AREA          	1    	0.4982 		11.506 		-3214.3

- O.HEIGHT        	1    	5.3541 		16.362 		-2943.9

- GLAZING.AREA    1   	13.3013 	24.309 		-2639.9

Step:  AIC=-3246.52

log.heatingLoad ~ S.AREA + W.AREA + O.HEIGHT + GLAZING.AREA +

GLAZE.AREA.DIS

Df 		Sum of Sq    	RSS     		AIC

<none>                        				11.033 		-3246.5

- S.AREA          	1    		0.0959 		11.129 		-3241.9

- GLAZE.AREA.DIS  1    		0.4505 		11.484 		-3217.8

- W.AREA          	1    		0.9421 		11.976 		-3185.6

- O.HEIGHT        	1   		 8.2720 	19.305 		-2818.9

- GLAZING.AREA    1   		13.3013 	24.335 		-2641.0

ANOVA Table:

Step 			Df     	Deviance 	Resid.Df 	Resid. Dev       AIC

1               		NA           NA       	760   		11.00704 	-3244.356

2 - ORIENTATION  	  1 	0.000675       	761   		11.00772 	-3246.308

3 - REL.COMPACT  	  1 	0.025721       	762   		11.03344 	-3246.516

Forward:

Start:  AIC=-1137.19

log.heatingLoad ~ 1

Df 		Sum of Sq     		RSS     		AIC

+ O.HEIGHT        	1   		140.401  		33.846 		-2393.7

+ S.AREA          	1    		78.716  		95.530 		-1596.8

+ REL.COMPACT     1    		72.331 			101.916 	-1547.1

+ W.AREA          	1    		34.903 			139.343 	-1306.9

+ GLAZING.AREA   1    		15.047 			159.200 	-1204.5

+ GLAZE.AREA.DIS 1    		 2.196 			172.050 	-1144.9

<none>                        					174.246 	-1137.2

+ ORIENTATION     1     		 0.001 			174.246 	-1135.2

Step:  AIC=-2393.68

log.heatingLoad ~ O.HEIGHT

Df 		Sum of Sq    	RSS     		AIC

+ GLAZING.AREA    1   		15.0469 	18.799 		-2843.3

+ W.AREA          	 1    		7.2190 		26.627 		-2575.9

+ S.AREA          	 1    		6.3728 		27.473 		-2551.9

+ REL.COMPACT      1    		5.3951 		28.451 		-2525.0

+ GLAZE.AREA.DIS 1    		2.1961 		31.650 		-2443.2

<none>                        				33.846 		-2393.7

+ ORIENTATION     1    		0.0007 		33.845 		-2391.7

Step:  AIC=-2843.27

log.heatingLoad ~ O.HEIGHT + GLAZING.AREA

Df 		Sum of Sq    	RSS     		AIC

+ W.AREA          	 1    		7.2190 	        	11.580 		-3213.4

+ S.AREA          	 1    		6.3728 		12.426 		-3159.2

+ REL.COMPACT     	 1    		5.3951 		13.404 		-3101.1

+ GLAZE.AREA.DIS 1    		0.4505 		18.348 		-2859.9

<none>                        				18.799 		-2843.3

+ ORIENTATION     1    		0.0007 		18.798 		-2841.3

Step:  AIC=-3213.39

log.heatingLoad ~ O.HEIGHT + GLAZING.AREA + W.AREA

Df 		Sum of Sq    	RSS     		AIC

+ GLAZE.AREA.DIS  1   		0.45053 	11.129 		-3241.9

+ S.AREA          	  1   		0.09594 	11.484 		-3217.8

+ REL.COMPACT       1                     0.06168           11.518 		-3215.5

<none>                        				11.580 		-3213.4

+ ORIENTATION     1   		0.00068 	11.579 		-3211.4

Step:  AIC=-3241.87

log.heatingLoad ~ O.HEIGHT + GLAZING.AREA + W.AREA + GLAZE.AREA.DIS

Df 		Sum of Sq    	RSS     		AIC

+ S.AREA       	1  		0.095935 	11.033 		-3246.5

+ REL.COMPACT  	1  		0.061684 	11.068 		-3244.1

<none>                     					11.129 		-3241.9

+ ORIENTATION  	1  		0.000676 	11.129 		-3239.9

Step:  AIC=-3246.52

log.heatingLoad ~ O.HEIGHT + GLAZING.AREA + W.AREA + GLAZE.AREA.DIS +

S.AREA

Df 		Sum of Sq    	RSS     		AIC

<none>                     					11.033 		-3246.5

+ REL.COMPACT  	1 		0.0257217 	11.008 		-3246.3

+ ORIENTATION  	1 		0.0006757 	11.033 		-3244.6

ANOVA Table:

Step  		Df     	Deviance 	Resid. Df 	Resid. Dev       	AIC

1          			NA           NA        	767  		174.24644 	-1137.189

2   + O.HEIGHT 	-1 	140.4006    	766   		33.84580 	-2393.677

3   + GLAZING.AREA 	-1  	15.04686    	765   		18.79894 	-2843.272

4   + W.AREA 		-1   	  7.21903      	764   		11.57990 	-3213.390

5   + GLAZE.AREA.DIS 	-1   	   0.45052     	763   		11.12937 	-3241.867

6         + S.AREA 	-1   	   0.09593     	762   		11.03344 	-3246.516

Both:

Start:  AIC=-3244.36

log.heatingLoad ~ REL.COMPACT + S.AREA + W.AREA + O.HEIGHT +

GLAZING.AREA + ORIENTATION + GLAZE.AREA.DIS

Df 		Sum of Sq   	 RSS     		AIC

- ORIENTATION     	1    		0.0007 		11.008 			-3246.3

- REL.COMPACT     	1    		0.0257 		11.033 			-3244.6

<none>                        				11.007 			-3244.4

- S.AREA          	1    		0.0600 		11.067 			-3242.2

- GLAZE.AREA.DIS  1    		0.4505 		11.458 			-3215.5

- W.AREA          	1    		0.4982 		11.505 			-3212.4

- O.HEIGHT        	1    		5.3541 		16.361 			-2941.9

- GLAZING.AREA    1   		13.3013 	24.308 			-2637.9

Step:  AIC=-3246.31

log.heatingLoad ~ REL.COMPACT + S.AREA + W.AREA + O.HEIGHT +

GLAZING.AREA + GLAZE.AREA.DIS

Df 		Sum of Sq    		RSS     		AIC

- REL.COMPACT     		1    		0.0257 			11.033 		-3246.5

<none>                        						11.008 		-3246.3

+ ORIENTATION     	1    		0.0007 			11.007 		-3244.4

- S.AREA          		1    		0.0600 			11.068 		-3244.1

- GLAZE.AREA.DIS  	1    		0.4505 			11.458 		-3217.5

- W.AREA          		1   		 0.4982 		11.506 		-3214.3

- O.HEIGHT       	 	1    		5.3541 			16.362	 	-2943.9

- GLAZING.AREA    		1   		13.3013 		24.309 		-2639.9

Step:  AIC=-3246.52

log.heatingLoad ~ S.AREA + W.AREA + O.HEIGHT + GLAZING.AREA +

GLAZE.AREA.DIS

Df 		Sum of Sq    		RSS     		AIC

<none>                        						11.033 		-3246.5

+ REL.COMPACT     		1    		0.0257 			11.008 		-3246.3

+ ORIENTATION     	1    		0.0007 			11.033 		-3244.6

- S.AREA          		1    		0.0959 			11.129 		-3241.9

- GLAZE.AREA.DIS  	1    		0.4505 			11.484 		-3217.8

- W.AREA          		1    		0.9421 			11.976 		-3185.6

- O.HEIGHT        		1    		8.2720 			19.305 		-2818.9

- GLAZING.AREA    		1   		13.3013 		24.335 		-2641.0

ANOVA Table:

Step 		Df     Deviance 		Resid. Df 	Resid. Dev       	AIC

1               		NA           NA      		 760   		11.00704 	-3244.356

2 - ORIENTATION  	1 	0.000675   	 761   		11.00772 	-3246.308

3 - REL.COMPACT  	1 	0.025721  	 762   		11.03344 	-3246.516

ROBUST REGRESSION:

Call: rlm(formula = log(HEATING.LOAD) ~ REL.COMPACT + W.AREA + O.HEIGHT +

GLAZING.AREA + GLAZE.AREA.DIS, data = file1.logresponse,

maxit = 100, method = "MM")

Residuals:

Min       1Q   Median       3Q      Max

-0.44906 -0.03766 -0.00613  0.05687  0.32271

Coefficients:

Value  		Std. Error 	t value

(Intercept)     		0.785  		0.074     	10.643

REL.COMPACT     	0.252  		0.085      	2.972

W.AREA         		 0.002  		0.000     	18.062

O.HEIGHT        	0.211  		0.005     	40.309

GLAZING.AREA   	 0.925  		0.023     	40.767

GLAZE.AREA.DIS -	0.002  		0.002     	-0.930

Residual standard error: 0.0576 on 762 degrees of freedom

VIF’s from robust regression:

REL.COMPACT         W.AREA       O.HEIGHT   GLAZING.AREA GLAZE.AREA.DIS

10.99          	 3.68          	11.61           	1.01           		1.01

VIF’s from best chosen model:

REL.COMPACT         W.AREA       O.HEIGHT   GLAZING.AREA GLAZE.AREA.DIS

9.25           	 3.16           	9.63           	1.05           		1.05